# Honors Project: Implementing XTEA on the DE2-115 with PS/2 Keyboard Input & LCD Output

Course: CSCI 4532 Hardware and Embedded Systems
Board: Terasic DE2-115 (Cyclone IV) – download the Cyclone® IV device support .qdz file
Peripherals: PS/2 Keyboard (input), 16×2 LCD (output)
Cipher: XTEA (64-bit block, 128-bit key, 32 cycles)

## 1) Learning Objectives

By the end of this project, you will be able to:

- Implement and integrate a round-based block cipher hardware core (XTEA) on FPGA.

- Interface the FPGA with a PS/2 keyboard using Set-2 scan codes and translate scan codes to ASCII.

- Drive a 16×2 character LCD and format binary blocks as hex strings for display.

- Validate a cryptographic design using known test vectors in simulation and on hardware.

- (Optional) Evaluate area/latency trade-offs; reason about protocol/IO design and basic side-channel considerations.

## 2) Background (short)

- XTEA is a lightweight 64-bit block cipher with a 128-bit key. A common implementation performs 32 cycles, each cycle comprising two Feistel operations, with a constant delta = 0x9E3779B9.

- The DE2-115 board provides a PS/2 port for keyboard input and a 16×2 LCD for easy display. The PS/2 keyboard uses a simple synchronous serial protocol delivering scan codes (Set-2), which you will map to ASCII (A–Z, 0–9, punctuation, Enter, Backspace).

- The LCD uses an HD44780-compatible interface and standard initialization (Function Set 0x38, Display ON 0x0C, Clear 0x01, Entry Mode 0x06).

You will enter the 128-bit key as 32 hex digits. For the 64-bit block, you may enter either 16 hex digits or 8 ASCII characters (hex is always displayed). The system then encrypts/decrypts and shows the 16-hex-character result on the second LCD line.

## 3) Files Provided

Download the package posted with this project (student_xtea_ps2_ascii.zip) and add all files to your Quartus project:

- top_xtea_ps2_lcd.v — integrates PS/2 keyboard, ASCII/HEX entry, XTEA core, LCD

- xtea_core.v — synthesizable 32-cycle XTEA (encrypt/decrypt)

- ps2_keyboard.v — PS/2 Set-2 receiver + scan-to-ASCII mapper (A–Z, 0–9, space, , . - /, Enter, Backspace)

- lcd_text.v — 16×2 LCD controller (8-bit data, RS/RW/E)

- hex_ascii.v, hex64_to_ascii16.v — helpers

- tb_xtea_core.v — self-checking testbench

- README.md — build notes and user instructions

Do not modify xtea_core.v interface unless instructed; you may add comments or instantiate it differently in the top level if you extend features.

## 4) Preparation

Complete before you start coding/building:

- Read & summarize (briefly, 1–2 paragraphs):

  - The XTEA round structure (what a “cycle” means, how sum and delta are used, why key indices are sum & 3 and (sum>>11) & 3).

- Pin Planning:

  - Identify the signal names we will use in Quartus:

    - Clock: CLOCK_50

    - PS/2: PS2_KBCLK, PS2_KBDAT

    - LCD: LCD_ON, LCD_BLON, LCD_EN, LCD_RS, LCD_RW, LCD_DATA[7..0]

  - Prepare to use DE2-115 System Builder or the class .qsf to establish correct pin assignments.

- Simulation setup:

  - Open tb_xtea_core.v and describe in your own words what the testbench checks and how it asserts pass/fail.

- Safety & Handling:

  - ESD awareness, proper power-up sequence, and safe cable handling for PS/2 and LCD connectors.

Submit these items as a PDF no later than 18 March, 2026.

## 5) Required Features (Functional Spec)

Your integrated design must support:

- Key entry: 32 hex digits (128-bit key).

- Block entry (plaintext/ciphertext): 16 hex digits (64-bit) or 8 ASCII characters (toggle M or use A/H to select ASCII/HEX).

- Commands:

  - K: select Key field

  - P: select Block field

  - M: toggle ASCII/HEX (block)

  - A: force ASCII mode; H: force HEX mode

  - E: Encrypt (when both key and block are complete)

  - D: Decrypt (when both key and block are complete)

  - C: Clear both fields

  - Backspace: delete last nibble (HEX) or last char (ASCII)

- LCD status:

  - Line 1 shows current field/mode and how many characters entered (e.g., HEX 16/16, ASCII 5/8, KEY 20/32).

  - Line 2 shows the current 64-bit block as 16 hex characters (after encryption/decryption and during editing).

## 6) Verification

### 6.1 Simulation (ModelSim)

Run the included testbench:

vlog xtea_core.v tb_xtea_core.v

vsim -c tb_xtea_core -do "run -all; quit"

Expected: Console prints “All XTEA tests passed.” The testbench checks four known vectors:

| # | Key (hex, 16 bytes) | Plaintext (hex, 8 bytes) | Ciphertext (hex, 8 bytes) |
| --- | --- | --- | --- |
| 1 | 000102030405060708090A0B0C0D0E0F | 4142434445464748 (“ABCDEFGH”) | 497DF3D072612CB5 |
| 2 | 000102030405060708090A0B0C0D0E0F | 4141414141414141 (“AAAAAAAA”) | E78F2D13744341D8 |
| 3 | 00000000000000000000000000000000 | 4142434445464748 | A0390589F8B8EFA5 |
| 4 | 00000000000000000000000000000000 | 4141414141414141 | ED23375A821A8C2D |

These vectors are drawn from widely used XTEA test suites (e.g., Go’s golang/crypto tests and community resources). In our design, 8-byte blocks are treated as big-endian when displayed as hex.

### 6.2 On-Board Demonstration

- Program the DE2-115 using Quartus.

- Plug in a PS/2 keyboard (or USB keyboard that supports PS/2 mode via passive adapter).

- Enter each key/plaintext pair above (HEX or ASCII) and press E. Confirm the LCD’s 16-hex result matches the expected ciphertext.

- Now set the block to the ciphertext, press D, and confirm you recover the original plaintext.

- Record a short video demonstrating at least two of the test vectors above first entering the plaintext to generate the ciphertext, then entering the generated ciphertext to produce the original plaintext.

## 7) Step-by-Step Procedure

### Step 1 — Create/Configure Project (Quartus)

- Create a new Cyclone IV project.

- Add all .v sources from the student ZIP.

- Use System Builder or the class .qsf to assign pins for CLOCK_50, PS2_KBCLK, PS2_KBDAT, and LCD_*.

- Compile and fix any warnings (no errors allowed).

Before moving forward: Project compiles; pins assigned; no combinational loops and no inferred latches reported.

### Step 2 — Simulate XTEA Core

- Run tb_xtea_core.v in ModelSim/Questa.

- Take a screenshot of the waveform at the moment done asserts on the last vector and include console output.

Before moving forward: Testbench passes; student explains what “32 cycles” means in XTEA and how sum is used.

Demonstrate Step 1 and Step 2 no later than 1 April, 2026.

### Step 3 — Hardware Bring-Up: PS/2

- Program the board. Type on the keyboard and observe that characters are accepted (you may blink an LED or add a debug print on LCD line 1 temporarily).

- Ensure Backspace, Enter, and hex letters A–F work for key/block entry.

### Step 4 — Encrypt/Decrypt on Board

- Enter Key #1 and Plaintext #1, press E; verify ciphertext on LCD (line 2).

- Press D using the resulting ciphertext to recover plaintext.

- Repeat with at least one more vector.

Demonstrate: On-board vectors match expected results twice (different keys/plaintexts).  Complete no later than 22 April, 2026.

### Step 5 — Clean Up & Notes

- Save your project; export a zipped project directory or git commit.

- Note any constraints you added and keep timing/resource reports.

## 8) Design Constraints & Expectations

- Verilog-2001; synchronous, active-high resets; no latches.

- XTEA core and I/O drivers must be synthesizable.

- Be mindful of clock domain (everything on CLOCK_50).

- Keep combinational fan-out reasonable; the design should meet timing comfortably at 50 MHz.

## 9) Deliverables (What to Submit)

Due: No later than 29 April, 2026.

- Report

  - Overview: your top-level block diagram and data/control flow (XTEA core, PS/2 RX, entry buffers, LCD).

  - Simulation evidence: screenshot(s) of waveforms and console log showing “All XTEA tests passed”.

  - Hardware evidence: photos or a short link to a video showing at least two vectors passing on the LCD (before/after).

  - Resource & timing summary: ALMs/LUTs and Fmax (Quartus) with 1–2 sentences of interpretation.

  - Reflections: 5–8 bullet points on what you learned, any bugs you fixed, and potential improvements.

- Code archive (zip or git link): your project files, including any minor UI edits (do not include generated db/ or incremental_db/ folders).

- Include the document submitted for 4) Preparation as an Appendix to this report.

## 10) Grading Rubric (100 pts)

- Functional correctness (40 pts)

  - Simulation vectors pass (20)

  - On-board demo matches vectors (20)

- Interface & UX (20 pts)

  - Key entry (32 hex) + block entry (HEX and ASCII), Backspace works, LCD updates (20)

- Design quality (25 pts)

  - Clean FSMs, synchronous reset, no latches; correct widths; readable comments (10)

  - Modularization and separation of concerns (core vs. I/O) (10)

  - Resource/timing note with interpretation (5)

- Demonstration (15 pts)

  - Live E/D with multiple inputs; clear explanation of XTEA’s 32 cycles and delta usage (15)

## 11) Tips & Troubleshooting

- PS/2 keyboard not registering?

  - Verify pin assignments and pull-ups; PS/2 Set-2 make codes should be observed (e.g., ‘A’ = 0x1C, ‘1’ = 0x16).

  - Remember break prefix 0xF0; the provided module ignores releases.

- LCD blank or garbled?

  - Ensure LCD_ON=1, LCD_BLON=1, LCD_RW=0, and initialization sequence is sent (Function Set → Display ON → Clear → Entry Mode).

- Wrong ciphertext?

  - Check byte order when entering data by hex; confirm you used 32 cycles and delta=0x9E3779B9.

  - Verify that the 64-bit block is displayed big-endian on the LCD (first byte appears leftmost).

## 12) What to Discuss in Your Report

Be sure to answer concisely:

- Why does XTEA often implement 32 cycles rather than 64?

- In your hardware, where are the state registers (v0, v1, sum), and when are they updated?

- How does your PS/2 receiver frame an 11-bit codeword (start, 8 data LSB-first, parity, stop)?

- How did you ensure your LCD writes are stable (RS/RW/E timing and ~microsecond tick)?

- What is the latency from start to done at 50 MHz, and how would you increase throughput?

## 13) Submission Checklist

- Preparation document, due 18 March, 2026

- Show 7. Step 1 — Create/Configure Project and 7. Step 2 — Simulate XTEA Core, due 1 April, 2026

- Show 7. Step 3 — Hardware Bring Up: PS/2 and 7. Step 4 — Encrypt/Decrypt on Board, due 22 April, 2026

- Full report, due 29 April, 2026

- Code archive (zip or repo link)